# Qualcomm KPI Logging

This fork adds --kpi support to output LTE key performance indicators (KPIs) to the log window.

## Usage

# USB mode - show KPIs in log

scat -t qc -u --kpi

# Serial mode

scat -t qc -s /dev/ttyUSB0 --kpi

# Dump file

scat -t qc -d capture.qmdl --kpi

When --kpi is used, the MAC layer is automatically enabled (if not already in --layer).

## KPI Output

When --kpi is used, the following are printed to the log:

### Cell Measurements

| Output | Description |
| --- | --- |
| LTE Primary Cell: | Serving cell (IDLE): EARFCN, PCI, RSRP, RSSI, RSRQ, priority (0–7, cell reselection layer priority from network) |
| LTE Primary Cell (Connected): | Serving cell (RRC_CONNECTED): PCI, DL RSRP, RSSI, RSRQ |
| Neighbor cell N: | Neighbor cell: EARFCN, PCI, RSRP, RSSI, RSRQ (signal KPIs; per-frequency priority is in SIBs, not in this diag log) |

The log mask now enables 0xB193 (Serving Cell Meas Response) and 0xB195 (Connected Mode Neighbor Meas) to try to get measurements in RRC_CONNECTED. The modem may still emit these less frequently than in IDLE. When --kpi is used and the modem does not report SCell in RRC_CONNECTED, the last known SCell (from IDLE or from a previous 0xB193) is re-printed at most every 2 seconds before KPI lines so that you still see EARFCN/PCI/RSRP in connected state.

### MAC Transport Block KPIs

| Field | Description |
| --- | --- |
| MCS | Modulation and Coding Scheme 0–31 (DL: derived from TBS+bandwidth per 36.213; UL: from grant) |
| TA | Timing Advance 0–63 (from MAC CE LCID 0x1D in DL MAC PDU; only in RRC_CONNECTED) |

DL MCS lines (X MHz BW MCS=Y) are throttled to at most one per 2 seconds per radio to avoid flooding the log during EPS (LTE) download, when the modem reports a new value on every DL transport block.

EPS throughput is computed from MAC transport block sizes (DL + UL). Every 1 second per radio, and only when RRC state is RRC_CONNECTED, a line is printed: LTE throughput: X.XX Mbps (combined DL+UL in the last 1 s window). Requires --kpi and MAC DL/UL TB logs (packet format v1).

In RRC_CONNECTED, UL MCS, TX power, and TA are combined into one throttled line (about once per second per radio): LTE KPI: UL MCS=…, TX power=… dBm, TA=…. Last-known values are used for each field; a field shows - if not yet received. The three individual lines (LTE KPI UL: MCS=…, LTE KPI TX: est. TX power=… dBm, LTE KPI: TA=…) are not printed in RRC_CONNECTED—only the combined line is.

### TX Power (from PHR)

| Field | Description |
| --- | --- |
| est. TX power | Estimated current PUSCH TX power (dBm), derived as Pcmax − PHR (Pcmax=23 dBm assumed). 23 dBm is suppressed in the log (phantom when no real UL/audio); +22 dBm and below print normally. |

This is current PUSCH power derived from the Power Headroom Report (PHR = Pcmax − P_PUSCH). LTE UL power control does reduce TX in good conditions (eNB power-controls the UE down when path loss is low) and increase TX in bad—so you’d expect good signal → lower TX, bad signal → higher TX. If you see the opposite (e.g. good → 23 dBm, bad → -7 dBm), common reasons are: (1) in poor signal the UE may not be in a state to use high power (RRC_SEARCH, no/small grants), so the reported current PUSCH power is low; (2) the modem’s PHR event may use a different encoding than standard 3GPP. So the value is “current PUSCH from PHR,” not “what power control would command” in all conditions. If the display looks opposite (good signal → 23 dBm, bad → -7 dBm), use --invert-tx-power so good signal shows lower TX and bad signal higher TX (matches power-control intuition).

### RACH (Random Access)

When the modem reports an LTE MAC RACH attempt (log 0xB062), a KPI line is printed:

LTE KPI RACH: result=success|failure, attempt=N, contention=0|1, preamble=X, preamble_power_dB=Y, ta=Z, tc_rnti=0x…, earfcn=E

- result — success (0x00) or failure

- attempt — RACH attempt number

- contention — Contention-based (1) or non-contention (0)

- preamble — Preamble index (Msg1); - if not present

- preamble_power_dB — Preamble power offset (raw); - if not present

- ta — Timing Advance from Msg2; - if not present

- tc_rnti — Temporary C-RNTI from Msg2; - if not present

- earfcn — DL EARFCN (from add-info when available); - otherwise

The line is emitted on both success and failure (and when Msg1/Msg2/Msg3 are incomplete). RACH data is also sent to Wireshark as GSMTAP MAC-LTE when the full 4-step sequence is present. When --json-udp-port is used, each RACH line is sent as a JSON object with type: "lte_rach" and the parsed fields (result, attempt, preamble, ta, tc_rnti, earfcn, etc.).

When you see RACH: The modem only reports RACH (log 0xB062) when a random access attempt occurs—e.g. initial attach, reconnection from idle, handover, or after radio recovery. To see RACH in the log and JSON, trigger a new connection (e.g. toggle airplane mode, or disable then re-enable mobile data) while scat is running with --kpi and --json-udp-port. RACH response packet versions 0x01, 0x31, and 0x32 are supported for different device formats.

### RRC State Changes

When the LTE RRC connection state changes, the new state and cause are logged:

| Output | Description |
| --- | --- |
| LTE RRC State: | New state: RRC_SEARCH, RRC_IDLE_NOT_CAMPED, RRC_IDLE_CAMPED, RRC_CONNECTING, RRC_CONNECTED, RRC_INACTIVE, RRC_CLOSING |
| LTE RRC State Cause: | Cause/trigger: RRCConnectionSetup, RRCConnectionRelease, RRCConnectionSetupComplete, etc. |

RRC_SEARCH (state 0) means the modem is in search (no RRC context yet)—e.g. after power-on or when signal is low but the modem can still decode (degraded link). RRC_INACTIVE (state 5) is connected-but-suspended (power saving).

RRC signalling messages (RRCConnectionRequest, Paging, etc.) are sent to Wireshark via GSMTAP—use Wireshark on the configured port to decode them.

When --kpi is used, event reporting is automatically enabled (required for RRC state).

### LTE handover events

When you run with --kpi, event reporting is already enabled (same as for RRC state), so LTE RRC DL/UL messages are reported. When a handover occurs you will see:

| Output | Description |
| --- | --- |
| LTE handover: RRCConnectionReconfiguration (network command) | Network sent handover command (RRC Reconfiguration with mobilityControlInfo). |
| LTE handover: RRCConnectionReconfigurationComplete (UE completed) | UE sent handover complete. |

RRC message types (RRCConnectionReconfiguration, RRCConnectionReconfigurationComplete, etc.) are also sent to GSMTAP for Wireshark. If your modem uses different message-type codes, you may see Unknown (xx) in the event; the handover lines above are emitted when the known codes (0x83 for Reconfiguration on DL, 0x83 for ReconfigurationComplete on UL) are reported.

### RRC FAILURE events

When --kpi is used, any Qualcomm event whose name contains FAILURE is printed to the log as RRC event: EVENT_…. Examples: EVENT_LTE_RRC_RADIO_LINK_FAILURE, EVENT_LTE_RRC_HO_FAILURE, EVENT_LTE_RRC_CELL_RESEL_FAILURE, EVENT_LTE_RRC_IRAT_HO_FROM_EUTRAN_FAILURE, EVENT_LTE_RRC_SIB_READ_FAILURE, and other FAILURE events from the fallback event list. These lines are also sent to JSON UDP when --json-udp-port is set.

### JSON over UDP

Use --json-udp-port PORT to send log output as JSON over UDP to localhost. Each line is converted to structured JSON and sent as a separate UDP packet. Use --no-gsmtap to disable GSMTAP output entirely so only JSON is sent (no traffic on port 4729).

python -m scat -t qc -s COM14 --kpi --dl-bandwidth 20 --json-udp-port 9999 --no-gsmtap

Start a receiver on port 9999 to consume the JSON (e.g. nc -ul 9999 or a Python script).

JSON structure (parsed when possible; otherwise type: "log" with raw message):

| type | Fields |
| --- | --- |
| lte_kpi_dl | bw_mhz, mcs |
| lte_kpi_ul | mcs |
| lte_kpi_tx | tx_power_dbm |
| lte_kpi_ta | ta (0–63) |
| lte_uplink_kpi | ul_mcs, tx_power_dbm, ta (each int or null if missing; emitted in RRC_CONNECTED instead of the three individual UL/TX/TA lines) |
| lte_rach | result (success¦failure), attempt, contention, preamble, preamble_power_dbm, ta, tc_rnti, earfcn (each numeric field int or null if missing) |
| lte_rrc_state | state |
| lte_rrc_state_cause | cause |
| lte_primary_cell | earfcn, pci, rsrp, rssi, rsrq, priority (optional, 0–7 when from SERVING_CELL_MEAS_AND_EVAL) |
| lte_scell_connected | pci, rsrp, rssi, rsrq |
| lte_ncell | cell_index, earfcn, pci, rsrp, rssi, rsrq |
| lte_throughput | mbps (combined DL+UL, 1 s window; only when RRC_CONNECTED) |
| log | message (unparsed) |

All objects include ts (ISO8601) and radio (0 or 1).

Example JSON (Primary Cell): {"ts":"2026-01-31T16:34:36.123456","radio":0,"type":"lte_primary_cell","earfcn":6300,"pci":106,"rsrp":-100,"rssi":-68,"rsrq":-15,"priority":5} (priority present when from SERVING_CELL_MEAS_AND_EVAL log)

Example JSON (TX power): {"ts":"2026-01-31T16:34:36.123456","radio":0,"type":"lte_kpi_tx","tx_power_dbm":8}

Example JSON (combined KPI): {"ts":"2026-01-31T16:34:36.123456","radio":0,"type":"lte_uplink_kpi","ul_mcs":25,"tx_power_dbm":23,"ta":1} (missing fields are null)

Example JSON (RACH): {"ts":"2026-01-31T16:34:36.123456","radio":0,"type":"lte_rach","result":"success","attempt":1,"contention":1,"preamble":10,"preamble_power_dbm":-104,"ta":7,"tc_rnti":18460,"earfcn":3749} (missing fields are null)

### Example Output

Radio 0: 20MHz BW MCS=20

Radio 0: LTE KPI: UL MCS=20, TX power=8 dBm, TA=1

Radio 0: LTE RRC State: RRC_CONNECTED

Radio 0: LTE RRC State Cause: RRCConnectionSetup

Radio 0: LTE KPI RACH: result=success, attempt=1, contention=1, preamble=5, preamble_power_dB=-96, ta=23, tc_rnti=0xf7d8, earfcn=-

Radio 0: LTE RRC State: RRC_IDLE_CAMPED

Radio 0: LTE RRC State Cause: RRCConnectionRelease

## Notes

- DL MCS: Derived from TBS and cell bandwidth (N_PRB) using 3GPP TS 36.213. If cell info (MIB, RRC, ML1) is not received, use --dl-bandwidth 20 to specify DL CC bandwidth in MHz (1.4, 3, 5, 10, 15, or 20).

- UL MCS: UL MCS is extracted from the 5 LSBs of the grant field (common convention in LTE DCI format 0). If your device uses a different grant encoding, the value may need adjustment.

- Device support: Uses LTE MAC transport block logs (0xB063 DL, 0xB064 UL). Packet format v1 is supported. Newer devices using v49 (0x31/0x32) format do not yet have KPI extraction.

- TX power: From LTE_ML1_PHR_REPORT event. Uses Pcmax=23 dBm; actual Pcmax may vary by device/band. PHR is reported when pathloss changes or periodically.