# Setting up SCAT on a new Windows PC

Use this when you have copied the scat-master folder (e.g. to C:\Users\<You>\OneDrive\Desktop\scat-master) on another Windows PC and want to run the same command line.

## 1. Install Python

- Install Python 3.10 or newer from python.org.

- During setup, enable “Add Python to PATH”.

- Confirm in a new Command Prompt or PowerShell:

python --version

You should see e.g. Python 3.12.x.

## 2. Install SCAT and all dependencies

Option A — Easiest: run the install script

- Open the scat-master folder in File Explorer.

- Double-click install.bat.

- It will install all dependencies from requirements.txt, then install SCAT in editable mode. Use the same Python that you use to run scat (e.g. from python.org).

Option B — Manual (Command Prompt or PowerShell)

cd C:\Users\<YourUsername>\OneDrive\Desktop\scat-master

Replace <YourUsername> with your Windows user name.

Install dependencies first, then SCAT:

python -m pip install -r requirements.txt

python -m pip install --editable "C:\Users\<YourUsername>\OneDrive\Desktop\scat-master"

Use the full path to your scat-master folder in the second command. If you get “requires 1 arg”, the path in quotes must be the argument to --editable.

Optional (faster CRC): after the above, run pip install libscrc.

## 3. Find the modem COM port (if using serial)

- Connect the modem (e.g. SIM7600) via USB.

- Open Device Manager → Ports (COM & LPT).

- Note the COM port for your modem (e.g. COM14). It may be different on this PC (e.g. COM3, COM5).

## 4. Run SCAT

From any directory (or from scat-master):

python -m scat -t qc -s COM14 --kpi --dl-bandwidth 20 --json-udp-port 9999

- Replace COM14 with the COM port you found in step 3.

- To disable GSMTAP and only send JSON: add --no-gsmtap:

python -m scat -t qc -s COM14 --kpi --dl-bandwidth 20 --json-udp-port 9999 --no-gsmtap

Note: If you use serial only (-s COMxx), you do not need PyUSB or libusb. If you use USB mode (--usb) or --list-usb, you need PyUSB and a Windows libusb backend (e.g. libusb-win32 or Zadig).

## Summary

| Step | Action |
| --- | --- |
| 1 | Install Python 3.10+ and add to PATH |
| 2 | Run install.bat in scat-master, or: pip install -r requirements.txt then pip install --editable "path" |
| 3 | Check Device Manager for the modem COM port |
| 4 | Run python -m scat -t qc -s COMxx --kpi --dl-bandwidth 20 --json-udp-port 9999 |

For more options and KPI details, see KPI_USAGE.md.